import numpy as np

from tkinter import *

import matplotlib.pyplot as plt

import matplotlib.animation as animation

from matplotlib.backends.backend_tkagg import FigureCanvasTkAgg

class FourBarApp:

# Notice we added l1, l2, l3, l4 to the initialization so it receives the numbers

def __init__(self, master, l1, l2, l3, l4):

self.L1 = l1

self.L2 = l2

self.L3 = l3

self.L4 = l4

# Plot Setup

self.fig = plt.figure(1)

plt.clf()

self.link, = plt.plot([], [], 'r-', linewidth=4)

self.joints, = plt.plot([], [], marker='o', ls="", markersize=10)

self.plot1 = FigureCanvasTkAgg(self.fig, master=master).get_tk_widget()

# Buttons

self.buton1 = Button(master, text="Start", command=self.start)

self.buton2 = Button(master, text="Stop", command=self.stop)

# Labels to show the lengths that were passed in

self.output1 = Label(master, text=f"L1: {self.L1}")

self.output2 = Label(master, text=f"L2: {self.L2}")

self.output3 = Label(master, text=f"L3: {self.L3}")

self.output4 = Label(master, text=f"L4: {self.L4}")

self.warning = Label(master, text="")

self.warning = Label(master, text="Grashof Mechanism", fg="blue", font=("Arial", 12, "bold"))

# UI Placement

self.plot1.place(x=0, y=0, height=500, width=800)

self.output1.place(x=100, y=500, height=20, width=100)

self.output2.place(x=250, y=500, height=20, width=100)

self.output3.place(x=400, y=500, height=20, width=100)

self.output4.place(x=550, y=500, height=20, width=100)

self.warning.place(x=100, y=550, height=20, width=600)

self.buton1.place(x=100, y=600, height=30, width=200)

self.buton2.place(x=500, y=600, height=30, width=200)

# Animation Setup

frames = self.generate_frames()

self.anim = animation.FuncAnimation(self.fig, self.animate, frames, interval=10, blit=False)

self.k = 0

def generate_frames(self):

L1, L2, L3, L4 = self.L1, self.L2, self.L3, self.L4

links = sorted([L1, L2, L3, L4])

S, L, P, Q = links[0], links[3], links[1], links[2]

# If Grashof AND L2 or L1 is the shortest, L2 can spin a full 360 degrees

if (S + L <= P + Q) and (S == L1 or S == L2):

return np.arange(0, 2*np.pi, 0.01)

# Otherwise (Double Rocker or Non-Grashof), calculate the swing limits!

else:

# Calculate cos(theta) at the folded and extended limits

cos_t1 = (L1**2 + L2**2 - (L3 - L4)**2) / (2 * L1 * L2)

cos_t2 = (L1**2 + L2**2 - (L3 + L4)**2) / (2 * L1 * L2)

# Clip values between -1 and 1 to prevent NumPy float errors

cos_t1 = max(-1.0, min(1.0, cos_t1))

cos_t2 = max(-1.0, min(1.0, cos_t2))

# Convert to radians

t_min = np.arccos(cos_t1)

t_max = np.arccos(cos_t2)

if t_min > t_max:

t_min, t_max = t_max, t_min

# Create an array that swings UP to the max, and then DOWN to the min

swing_up = np.arange(t_min, t_max, 0.02)

swing_down = np.arange(t_max, t_min, -0.02)

return np.concatenate((swing_up, swing_down))

def calculate(self, theta2):

L1, L2, L3, L4 = self.L1, self.L2, self.L3, self.L4

BD = np.sqrt(L1**2 + L2**2 - 2*L1*L2*np.cos(theta2))

cos_alfa = (L3**2 + L4**2 - BD**2) / (2*L3*L4)

cos_alfa = np.clip(cos_alfa, -1.0, 1.0) # Forces imperfect math strictly into the [-1, 1] range

alfa = np.arccos(cos_alfa)

#alfa = np.arccos((L3**2 + L4**2 - BD**2) / (2*L3*L4)) # [cite: 66]

theta3 = 2*np.arctan((-L2*np.sin(theta2) + L4*np.sin(alfa)) / (L1 + L3 - L2*np.cos(theta2) - L4*np.cos(alfa)))

theta4 = 2*np.arctan((L2*np.sin(theta2) - L3*np.sin(alfa)) / (L4 - L1 + L2*np.cos(theta2) - L3*np.cos(alfa)))

A = [0, 0]

B = [L2*np.cos(theta2), L2*np.sin(theta2)]

C = [L2*np.cos(theta2) + L3*np.cos(theta3), L2*np.sin(theta2) + L3*np.sin(theta3)]

D = [L1, 0]

return BD, alfa, theta3, theta4, A, B, C, D

def axis_setter(self):

# 1. Simulate the entire movement to get all coordinates

t2 = self.generate_frames()

BD, alfa, theta3, theta4, A, B, C, D = self.calculate(t2)

# 2. Gather every single X and Y coordinate the joints will ever touch

# A is always [0,0] and D is always [L1, 0]

all_x = np.concatenate(([0, self.L1], B[0], C[0]))

all_y = np.concatenate(([0, 0], B[1], C[1]))

# 3. Find the absolute extremes (nanmax/nanmin ignore math errors from extreme angles)

max_x = np.nanmax(all_x)

min_x = np.nanmin(all_x)

max_y = np.nanmax(all_y)

min_y = np.nanmin(all_y)

# 4. Create a dynamic padding/margin so the mechanism never touches the edge

# We base the padding on the size of the largest link

padding = max(self.L1, self.L2, self.L3, self.L4) * 0.2

upper_x = max_x + padding

lower_x = min_x - padding

upper_y = max_y + padding

lower_y = min_y - padding

return upper_x, lower_x, upper_y, lower_y

def start(self):

try:

upper_x, lower_x, upper_y, lower_y = self.axis_setter()

plt.axis([lower_x, upper_x, lower_y, upper_y])

plt.gca().set_aspect('equal', adjustable='box')

self.k = 1

self.anim.event_source.start()

self.warning.config(text="Grashof Mechanism", background="White")

except:

self.warning.config(text="Math error during rotation.", background="red", font=("Courier", 14))

self.stop()

return self.k

def stop(self):

self.k = 0

return self.k

def animate(self, theta2):

if self.k == 0:

self.anim.event_source.stop()

else:

BD, alfa, theta3, theta4, A, B, C, D = self.calculate(theta2)

x = [A[0], B[0], C[0], D[0]]

y = [A[1], B[1], C[1], D[1]]

self.link.set_data(x, y)

self.joints.set_data(x, y)

return self.link, self.joints

# This is the function we will call from your main file!

def launch_animation(l1, l2, l3, l4):

window = Toplevel() # Toplevel makes it a popup child window rather than a primary root window

window.geometry("800x800")

window.title("Four Bar Animation")

app = FourBarApp(window, l1, l2, l3, l4) # Passes the variables into the App